TECHNICAL DATA SHEET

PRODUCT NAME: POLTEM EKO WAX

PRODUCT DESCRIPTION

Poltem Eko Wax is a cream wax and using to release the polyester and GRP molding products from the moulds.

PRODUCT PROPERTIES

| Wax Base | Paraffin Hidrocarbon |
| --- | --- |
| Solvent Base | Aliphatic hydrocarbons |
| Appearance | Cream paste |

APPLICATION

On the used moulds, apply onto the all mould surface by circular motions and wait for 5-10 minutes to obtain a thin film. After that polish with a piece of clean dry cotton fabric. Keep stand it for 15-20 minutes in order to get dry. Finally wipe and polish with a clean cotton fabrik.

Poltem economic wax is compatible with PVA. After using Poltem eko wax, the finish product can be painted (or topcoat application).

You can obtain up to 6 series production with Poltem economic wax application on the glossy and low risk level moulds.

PACKAGING

The products are packaged in 1 kg tin boxes and 12 tin boxes are packaged in a shrink box.

| Net product weight (g) | 800 |
| --- | --- |
| Gross product weight (g) | 900 |

STORAGE

Keep the product in a dark and dry place at temperatures between 5-20 °C. After using keep it closed. Keep away from flames and electrical sparks. No smoking near the products.